Linux Networking & Commands

A Complete Study Reference — From Basics to Projects

# 1. Linux Basic Commands

All work is done in a Linux terminal (Ubuntu/Debian based). These are the foundational commands needed before networking.

## 1.1 File & Directory Commands

ls

→  List files in current directory

ls -la

→  List all files including hidden ones with details

pwd

→  Print Working Directory — shows current location

cd <folder>

→  Change directory

cd ..

→  Go one directory up

mkdir <name>

→  Create a new directory

rm <file>

→  Remove a file

rm -rf <folder>

→  Force remove a folder and everything inside

cp <src> <dest>

→  Copy a file

mv <src> <dest>

→  Move or rename a file

cat <file>

→  View contents of a file

touch <file>

→  Create an empty file

## 1.2 File Permissions

Every file in Linux has read (r), write (w), and execute (x) permissions for owner, group, and others.

chmod +x <file>

→  Add execute permission to a file (+x = add execute, -x = remove)

chmod 755 <file>

→  Set permissions using numeric notation (7=rwx, 5=r-x)

ls -l

→  View permissions of all files in current directory

## 1.3 Text Editor — nano

nano is a simple terminal-based text editor used to create and edit files directly from the command line.

nano <filename>

→  Open or create a file in nano editor

nano network_check.sh

→  Example: create a shell script file

Inside nano:

- Ctrl + O  →  Save the file (Write Out)

- Ctrl + X  →  Exit nano

- Ctrl + K  →  Cut a line

- Ctrl + U  →  Paste a line

## 1.4 Bash Scripting

A Bash script is a file containing multiple Linux commands that run together as a program.

nano network_check.sh

→  Step 1: Create the script file

chmod +x network_check.sh

→  Step 2: Give execute permission

./network_check.sh

→  Step 3: Run the script

The ./ before the script name tells the shell to look for the file in the current directory.

## 1.5 Package Management

sudo apt update

→  Refresh the list of available packages

sudo apt install <pkg>

→  Install a package

sudo apt remove <pkg>

→  Uninstall a package

sudo apt upgrade

→  Upgrade all installed packages

# 2. How Hotspot Works

A hotspot shares one device's internet connection with other devices over Wi-Fi. The device creating the hotspot acts as a router — it uses NAT (Network Address Translation) to let connected devices access the internet through its connection.

### Flow:

- Your phone/laptop connects to the internet (via mobile data or Ethernet)

- Hotspot is enabled — the device creates a Wi-Fi network

- Other devices connect to that Wi-Fi

- The hotspot device routes traffic between connected devices and the internet

# 3. Network Connection Checking Commands

## 3.1 ping — Test Network Reachability

ping is a diagnostic tool to check if a device or host is reachable over a network and to measure the Round Trip Time (RTT) of data packets.

ping -c 4 google.com

→  Send 4 packets to google.com using domain name (-c = count)

ping -c 4 8.8.8.8

→  Send 4 packets to Google's IP address directly

Difference: First uses DNS to resolve google.com → IP. Second bypasses DNS and hits the IP directly. If the first fails but second works, DNS may be the issue.

## 3.2 traceroute — Trace the Network Path

traceroute shows every router (hop) that data passes through from your device to the destination, along with the delay at each step.

traceroute google.com

→  Trace the route to google.com showing all hops

Useful for finding where a connection is slow or breaking down.

## 3.3 nslookup — DNS Lookup Tool

DNS (Domain Name System) converts human-readable names like google.com into IP addresses like 142.250.x.x so computers can communicate.

nslookup is used to find the IP address of a domain or get DNS-related information.

nslookup google.com

→  Find the IP address for google.com

If DNS is not working, the browser cannot resolve website names and the internet won't open websites even if the network connection is fine.

## 3.4 curl — Transfer Data / Test APIs

curl is a command-line tool to transfer data from or to a server using protocols like HTTP and HTTPS. Commonly used to test APIs and network connectivity.

curl ifconfig.me

→  Get your public IP address (the IP visible to the internet)

curl http://example.com

→  Fetch the HTML of a website

curl -I http://example.com

→  Fetch only the HTTP headers (useful for testing server response)

## 3.5 ip addr show — View IP Addresses

Linux command to display IP address details of all network interfaces on your system (replaces the older ifconfig command).

ip addr show

→  Show all network interfaces with their IP addresses

ip addr show eth0

→  Show details for a specific interface (eth0 = Ethernet)

ip route show

→  Show the routing table — how traffic is directed

# 4. DHCP — Dynamic Host Configuration Protocol

DHCP automatically assigns IP addresses and network configuration to devices when they connect to a network. Without DHCP, you would need to manually configure the IP address on every device.

## 4.1 DHCP Components

- DHCP Server — the router or a dedicated server that gives out IP addresses

- DHCP Client — any device requesting network configuration (laptop, phone, etc.)

- Client Port — 68

- Server Port — 67

- Protocol Used — UDP (fast, connectionless, supports broadcast)

## 4.2 DORA Process — How IP is Assigned

DHCP uses a 4-step process called DORA to obtain an IP address:

D — Discover

→  Client broadcasts a message: 'Is there any DHCP server available?'

O — Offer

→  Server responds: 'I can offer you IP 192.168.1.10'

R — Request

→  Client replies: 'Yes, I would like that IP address'

A — Acknowledge

→  Server confirms: 'IP 192.168.1.10 is yours for a lease time'

## 4.3 UDP vs TCP

UDP (User Datagram Protocol)

- Fast, connectionless — does not wait for confirmation

- Supports broadcast — can send to all devices on network at once

- Used by DHCP because the device has no IP yet and needs to broadcast

TCP (Transmission Control Protocol)

- Connection-oriented — establishes connection first (handshake)

- Ensures data is received and acknowledged

- Used for reliable data transfer (web browsing, file transfer, SSH)

# 5. SSH — Secure Shell

SSH lets you remotely log in and control another Linux machine or router securely over the network. All data is encrypted.

## 5.1 Basic SSH Commands

ssh user@<ip-address>

→  Connect to a remote machine using username and IP

ssh root@192.168.1.1

→  Connect to a router (192.168.1.1 is the common default gateway IP)

exit

→  Disconnect and close the SSH session

ssh -p 2222 user@<ip>

→  Connect on a custom port (default is 22)

ssh -i ~/.ssh/id_rsa user@<ip>

→  Connect using a specific private key file

## 5.2 SSH Key Generation

ssh-keygen -t rsa -b 4096

→  Generate a new RSA key pair (public + private)

ssh-copy-id user@<ip>

→  Copy your public key to remote machine for passwordless login

cat ~/.ssh/id_rsa.pub

→  View your public key

## 5.3 SCP — Secure Copy (File Transfer over SSH)

scp root@192.168.1.1:/etc/config/network ./network_backup

→  Copy a file FROM remote router TO local machine

scp ./network_config root@192.168.1.1:/etc/config/network

→  Upload a file FROM local machine TO remote router

scp -r user@<ip>:/remote/folder ./local/

→  Recursively copy an entire folder from remote

# 6. OpenWrt — Router Firmware

OpenWrt is a Linux-based open-source firmware for routers. It replaces the default router software and gives full control over routing, firewall, wireless settings, VPN, and more. You access it via SSH (terminal) or LuCI (web interface).

## 6.1 Connecting to OpenWrt Router

ssh root@192.168.1.1

→  SSH into the router (default IP, no password on fresh install)

ping 192.168.1.1

→  Check if the router is reachable before connecting

## 6.2 OpenWrt Network & Service Commands

/etc/init.d/network restart

→  Restart the network service to apply config changes

/etc/init.d/firewall restart

→  Restart the firewall service

/etc/init.d/dnsmasq restart

→  Restart the DNS/DHCP service

service network restart

→  Alternative way to restart network service

reboot

→  Reboot the router

## 6.3 Viewing Router Config Files

cat /etc/config/network

→  View the network configuration file

cat /etc/config/wireless

→  View the wireless (Wi-Fi) configuration file

cat /etc/config/firewall

→  View the firewall rules

cat /etc/config/system

→  View system config including hostname and NTP settings

cat /etc/config/dhcp

→  View DHCP and DNS settings

## 6.4 OpenWrt Package Management (opkg)

opkg update

→  Update the list of available packages

opkg install <package>

→  Install a package on OpenWrt

opkg remove <package>

→  Remove a package

opkg list-installed

→  Show all installed packages

## 6.5 Default Restore / Reset

firstboot

→  Reset OpenWrt to factory defaults

firstboot && reboot now

→  Reset and immediately reboot

# 7. UCI — Unified Configuration Interface

UCI (Unified Configuration Interface) is OpenWrt's command-line system for reading and modifying configuration files stored in /etc/config/. Instead of directly editing config files, UCI provides clean commands to get, set, and save settings. Changes are only applied after committing and restarting the relevant service.

## 7.1 Viewing Configuration

uci show

→  Show the entire system configuration (all sections)

uci show network

→  Show only the network configuration

uci show wireless

→  Show only the wireless (Wi-Fi) configuration

uci show firewall

→  Show the firewall configuration

uci show system

→  Show system configuration (hostname, NTP, etc.)

uci show dhcp

→  Show DHCP and DNS configuration

uci show network.lan

→  Show only the LAN interface section

uci show network.wan

→  Show only the WAN interface section

uci get network.lan.ipaddr

→  Get a single specific value — the LAN IP address

uci get network.lan.netmask

→  Get the LAN subnet mask

uci get system.@system[0].hostname

→  Get the router hostname

## 7.2 Setting / Changing Configuration

uci set network.lan.ipaddr='192.168.1.1'

→  Change the LAN IP address

uci set network.lan.netmask='255.255.255.0'

→  Change the LAN subnet mask

uci set network.lan.gateway='192.168.1.254'

→  Set the default gateway

uci set network.lan.dns='8.8.8.8'

→  Set the DNS server

uci set network.wan.proto='dhcp'

→  Set WAN protocol to DHCP

uci set network.wan.proto='static'

→  Set WAN protocol to static IP

uci set wireless.@wifi-iface[0].ssid='MyNetwork'

→  Change the Wi-Fi SSID (network name)

uci set wireless.@wifi-iface[0].key='mypassword'

→  Change the Wi-Fi password

uci set wireless.@wifi-iface[0].encryption='psk2'

→  Set Wi-Fi encryption to WPA2

uci set wireless.@wifi-device[0].disabled='0'

→  Enable the Wi-Fi radio (0=enabled, 1=disabled)

uci set system.@system[0].hostname='MyRouter'

→  Change the router hostname

uci set system.@system[0].timezone='IST-5:30'

→  Set timezone to India Standard Time

## 7.3 Adding New Configuration Sections

uci add network interface

→  Add a new network interface section

uci add firewall rule

→  Add a new firewall rule section

uci add_list network.lan.dns='8.8.4.4'

→  Add an item to a list value (e.g., second DNS)

## 7.4 Deleting Configuration

uci delete network.lan.gateway

→  Delete a specific option

uci delete network.@interface[1]

→  Delete an entire interface section

uci del_list network.lan.dns='8.8.4.4'

→  Remove one item from a list value

## 7.5 Saving & Applying Changes

uci commit

→  Commit (save) ALL pending changes across all configs

uci commit network

→  Commit only network changes

uci commit wireless

→  Commit only wireless changes

uci commit firewall

→  Commit only firewall changes

After committing, always restart the relevant service to apply the changes:

/etc/init.d/network restart

→  Apply network changes

/etc/init.d/wireless restart

→  Apply wireless changes

wifi

→  Shortcut to reload wireless config

## 7.6 Reverting Uncommitted Changes

uci revert network

→  Undo all uncommitted changes to network config

uci revert network.lan.ipaddr

→  Undo a specific uncommitted change

## 7.7 Exporting & Importing Config

uci export network

→  Export network config in UCI batch format

uci export > /tmp/full_backup.uci

→  Export full configuration to a file

uci import < /tmp/full_backup.uci

→  Import configuration from a file

# 8. ubus — Micro Bus Architecture

ubus is OpenWrt's inter-process communication (IPC) system. It acts as a message bus that allows different system processes and services to talk to each other. Using ubus from the command line, you can call functions on running services, subscribe to events, and query system state in real time — without needing to restart anything.

## 8.1 Listing & Exploring Objects

ubus list

→  List all available ubus objects (services registered on the bus)

ubus list network.*

→  List all network-related ubus objects

ubus list system

→  Show the system ubus object

## 8.2 Inspecting Available Methods

ubus -v list

→  List all objects AND their available methods with parameters

ubus -v list network.interface

→  Show all methods available on the network.interface object

ubus -v list system

→  Show all methods available on the system object

## 8.3 Network Interface Commands

ubus call network.interface.lan status

→  Get full status of LAN interface (IP, MAC, uptime, etc.)

ubus call network.interface.wan status

→  Get full status of WAN interface

ubus call network.interface.loopback status

→  Get loopback interface status

ubus call network.interface dump

→  Dump all interface information at once

ubus call network.interface.lan up

→  Bring LAN interface UP

ubus call network.interface.lan down

→  Bring LAN interface DOWN

## 8.4 System Commands

ubus call system board

→  Get hardware info: board name, model, kernel version

ubus call system info

→  Get system info: uptime, memory usage, load average

ubus call system reboot

→  Reboot the router via ubus

## 8.5 Service Management via ubus

ubus call service list

→  List all running services on the system

ubus call service list '{"name":"network"}'

→  Get status of a specific service (network)

ubus call service start '{"name":"network"}'

→  Start a service

ubus call service stop '{"name":"network"}'

→  Stop a service

## 8.6 Wireless Commands

ubus call network.wireless status

→  Get wireless radio and interface status

ubus call network.wireless notify

→  Trigger wireless notification event

## 8.7 DHCP / Hostapd via ubus

ubus call hostapd.<iface> get_clients

→  Get list of connected Wi-Fi clients on an interface

ubus call hostapd.<iface> del_client '{"addr":"AA:BB:CC:DD:EE:FF"}'

→  Disconnect a specific Wi-Fi client by MAC address

## 8.8 Subscribing to Events

ubus subscribe network.interface

→  Subscribe and listen to network interface events in real time

ubus wait_for network.interface

→  Wait until the network.interface object becomes available

## 8.9 Using ubus with JSON Output

ubus call system info | jsonfilter -e '@.memory'

→  Filter only memory info from system info output

ubus -S call network.interface.lan status

→  -S flag: simplified/raw output for scripting

# 9. NTP — Network Time Protocol

NTP synchronizes the clock on your device/router with internet time servers. Accurate time is important for SSL certificates, logs, and VPNs to function correctly.

ntpdate pool.ntp.org

→  Manually sync time from NTP server

timedatectl status

→  Check current time and NTP sync status

timedatectl set-ntp true

→  Enable automatic NTP synchronization

cat /etc/config/system

→  View NTP configuration on OpenWrt

# 10. VPN — Virtual Private Network

A VPN creates an encrypted tunnel between two points over the internet, making traffic private and secure. OpenVPN is an open-source VPN solution widely used in Linux.

## 7.1 OpenVPN — Installation & Setup on Linux

sudo apt install openvpn

→  Install OpenVPN

sudo openvpn --config client.ovpn

→  Connect to VPN using a config file

sudo systemctl start openvpn@client

→  Start VPN as a system service

sudo systemctl enable openvpn@client

→  Enable VPN to auto-start on boot

sudo systemctl status openvpn@client

→  Check VPN service status

## 7.2 OpenVPN — Creating Server & Client Certificates

PKI (Public Key Infrastructure) certificates are used to authenticate the server and clients. Easy-RSA is the tool used to manage these.

### Server Side — Initialize PKI & Create CA

apt install easy-rsa

→  Install Easy-RSA certificate tool

make-cadir ~/openvpn-ca

→  Create directory with Easy-RSA scripts

cd ~/openvpn-ca

→  Enter the CA directory

./easyrsa init-pki

→  Initialize PKI directory structure

./easyrsa build-ca

→  Build the Certificate Authority (CA)

./easyrsa gen-req server nopass

→  Generate server certificate request

./easyrsa sign-req server server

→  Sign the server certificate with CA

./easyrsa gen-dh

→  Generate Diffie-Hellman parameters (used for key exchange)

### Client Side — Create Client Certificate

./easyrsa gen-req client1 nopass

→  Generate client certificate request

./easyrsa sign-req client client1

→  Sign client certificate with CA

The resulting .crt, .key, and ca.crt files are distributed to the client for connecting.

## 7.3 VPN Network Creation & Connection

ip tun add tun0 mode tun

→  Create a TUN (virtual network) interface

ip link set tun0 up

→  Bring up the TUN interface

ip addr add 10.8.0.1/24 dev tun0

→  Assign IP to TUN interface

ip route add 10.8.0.0/24 via 10.8.0.1

→  Add route for VPN subnet

# 11. Wireshark — Network Packet Analysis

Wireshark is a GUI tool to capture and analyze network packets in real time. It helps visualize TCP/UDP message flow.

## 8.1 Installing and Opening Wireshark

sudo apt install wireshark

→  Install Wireshark

sudo wireshark

→  Open Wireshark with root permission (needed to capture packets)

## 8.2 TCP Protocol — Capturing the 3-Way Handshake

TCP uses a 3-way handshake before sending data: SYN → SYN-ACK → ACK

### Commands to generate TCP traffic for Wireshark to capture:

curl http://example.com

→  Generate HTTP (TCP) traffic

nc -lvp 1234

→  Start a netcat TCP listener on port 1234

nc <server-ip> 1234

→  Connect to the TCP listener from another terminal

### Wireshark filters for TCP:

tcp

→  Show all TCP packets

tcp.port == 80

→  Filter by port 80 (HTTP)

tcp.flags.syn == 1

→  Show only SYN packets (connection starts)

## 8.3 UDP Protocol — Capturing Message Flow

UDP does not establish a connection — it simply sends packets. Used to see message flow without handshake.

nc -u -lvp 5005

→  Start UDP listener on port 5005

nc -u <server-ip> 5005

→  Send UDP messages to listener

echo 'hello' | nc -u <ip> 5005

→  Send a single UDP message via command

### Wireshark filter for UDP:

udp

→  Show all UDP packets

udp.port == 5005

→  Filter by specific UDP port

# 12. Docker — Containerization

Docker runs applications in isolated containers — lightweight virtual environments that contain everything needed to run an app.

## 9.1 Installation

sudo apt install docker.io

→  Install Docker

sudo systemctl start docker

→  Start Docker service

sudo systemctl enable docker

→  Enable Docker to start on boot

## 9.2 Basic Docker Commands

docker --version

→  Check Docker version

docker pull <image>

→  Download an image from Docker Hub

docker images

→  List all downloaded images

docker ps

→  Show running containers

docker ps -a

→  Show all containers including stopped ones

docker run <image>

→  Create and start a container from an image

docker run -it ubuntu bash

→  Run Ubuntu container interactively with bash shell

docker run -d -p 8080:80 nginx

→  Run nginx in background, map port 8080 → 80

docker stop <container_id>

→  Stop a running container

docker rm <container_id>

→  Remove a stopped container

docker exec -it <id> bash

→  Open shell inside a running container

## 9.3 Dockerfile — Build Custom Images

nano Dockerfile

→  Create a Dockerfile

docker build -t myapp .

→  Build image from Dockerfile in current directory

docker run myapp

→  Run your custom built image

# 13. Git — Version Control

Git tracks changes to code and files, enabling collaboration and version history.

## 10.1 Setup

sudo apt install git

→  Install Git

git config --global user.name 'Your Name'

→  Set your name

git config --global user.email 'you@email.com'

→  Set your email

## 10.2 Basic Git Workflow

git init

→  Initialize a new Git repository

git clone <url>

→  Download (clone) an existing repository

git status

→  Check what files have changed

git add <file>

→  Stage a file for commit

git add .

→  Stage all changed files

git commit -m 'message'

→  Save staged changes with a description

git push origin main

→  Upload commits to remote repository (GitHub)

git pull

→  Download latest changes from remote

git log

→  View commit history

git branch

→  List all branches

git checkout -b <branch>

→  Create and switch to a new branch

# 14. CAPWAP Project Commands

CAPWAP (Control And Provisioning of Wireless Access Points) is a protocol that allows a Wireless LAN Controller (WLC) to manage multiple Access Points (APs) centrally.

## 11.1 Project Setup & Environment

git clone <capwap-repo-url>

→  Clone the CAPWAP project repository

cd capwap

→  Enter the project directory

docker build -t capwap-wlc .

→  Build Docker image for WLC

docker-compose up

→  Start all CAPWAP services defined in docker-compose.yml

docker-compose down

→  Stop all CAPWAP services

## 11.2 Network Setup for CAPWAP

docker network create capwap-net

→  Create a Docker network for CAPWAP

docker network ls

→  List all Docker networks

docker network inspect capwap-net

→  View details of the CAPWAP network

## 11.3 Running & Testing CAPWAP

docker exec -it wlc bash

→  Access WLC container shell

docker exec -it ap1 bash

→  Access Access Point container shell

tcpdump -i any -w capwap.pcap

→  Capture all packets to a file for Wireshark analysis

tcpdump -i eth0 port 5246

→  Capture CAPWAP control traffic (port 5246)

tcpdump -i eth0 port 5247

→  Capture CAPWAP data traffic (port 5247)

CAPWAP Ports:

- UDP 5246 — CAPWAP Control channel (WLC ↔ AP management messages)

- UDP 5247 — CAPWAP Data channel (actual wireless user data)

## 11.4 Checking Connectivity in Project

ping <wlc-ip>

→  Test AP can reach the WLC

ip addr show

→  Verify IP addresses assigned in containers

netstat -an | grep 5246

→  Check if CAPWAP control port is listening

# 15. OpenVPN — POC (Proof of Concept) Project

This project demonstrated setting up a full OpenVPN server-client architecture on Linux with certificate-based authentication.

## 12.1 Server Setup

sudo apt install openvpn easy-rsa

→  Install OpenVPN and certificate tools

cp -r /usr/share/easy-rsa ~/easy-rsa

→  Copy Easy-RSA to home directory

./easyrsa init-pki

→  Initialize PKI

./easyrsa build-ca nopass

→  Build Certificate Authority without password

./easyrsa gen-req server nopass

→  Generate server key pair

./easyrsa sign-req server server

→  Sign server certificate

./easyrsa gen-dh

→  Generate Diffie-Hellman key

openvpn --genkey secret ta.key

→  Generate TLS auth key (extra security layer)

## 12.2 Client Setup

./easyrsa gen-req client1 nopass

→  Generate client key pair

./easyrsa sign-req client client1

→  Sign client certificate

Copy these files to the client machine: ca.crt, client1.crt, client1.key, ta.key

## 12.3 Starting OpenVPN

sudo openvpn --config /etc/openvpn/server.conf

→  Start OpenVPN server

sudo openvpn --config client.ovpn

→  Connect client to VPN

ip addr show tun0

→  Verify VPN tunnel interface is up

ping 10.8.0.1

→  Test connectivity to VPN server through tunnel

Linux Networking Study Reference  •  All commands executed on Ubuntu/Debian Linux